# ANNEX IV: Budget breakdown (Model financial offer)

Contract title: ”Procurement of communal special vehicle for transport of selected waste material and Hydraulic automatic continuous press”

Lot 1: 		Hydraulic automatic continuous press

Page No 1 [of 4]

PUBLICATION REFERENCE: BIOREAL 808/3 		NAME OF TENDERER: <name>

| A |  |  |  | C |  | D |  | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number |  | Quantity |  | specifications offered (incl brand/model) |  | Unit costs with delivery DDP Prilep [MKD] |  | total [MKD] |
| 1 | 1 |  | Hydraulic automatic continuous press with dosing equipment for all types of waste recyclables Manufacturer’s name: Product type, model: 1.1 Technical specification for the hydraulic automatic continuous press a) Baling material: PET, metal, foil, nylon, paper, cardboard, aluminum, RDF b) Capacity: minimum 4 t / h c) Installed power: minimum 15-maximized 22KW d) Mode of operation: automatic / manual e) Maximum pressure: minimum 200 maximum 240 bars f) Pressure force: minimum 500 KN or 50 t g) Number of bales per hour : minimum 10 h) Cycle time: minimum 20 sec, maximum 48 sec i) Special accessory: (perforator) drill with steel tufts (chromed) for drilling of PET packaging: length according to the press and trough characteristics j) The width of the press: minimum 4000mm k) Height of the press: according to the technological project l) Length of the press: minimum 7200 mm m) Weight of the press: minimum 10 000 kg. n) Bale size: minimum 200mm x750 mm o) Bale weight: minimum 200 kg maximum 600 kg p) Bale length: minimum 1000 max. 2500 mm q) Bale tying: Automatic with soft wire 2,5 - 3 mm r) Number of bale ties: 4 horizontal s) Bale binding mode: automatic t) Capacity: minimum 2 tones / hour (depending on the material) u) Dosing equipment v) Filling method: Conveyor belt, trolley w) Conveyor belt length: minimum 5500 x 1200/100 nm x) Material for making a press filler kit: Steel Hardox 1.2 Additional elements for Hydraulic automatic continuous press a) Equipment installation: done by the supplier b) Deadline for equipment installation: 10 days from the day of arrival c) Equipment catalogue d) Quality guarantee: 24 months e) Written illustrated manual for using the press f) 1000 liters Hydrol are provided during installation g) Written illustrated instructions for using hydrol h) Certificate of Quality ISO 9001 , 14001 1.3 Training of end-users from PCE "Komunalec" – Prilep - The Beneficiary’s personnel will be trained in the use of the system, and the operation and maintenance of all the equipment that will be delivered under this contract in 15 days from the date of equipment installation - The training will provide the 6 Beneficiary personnel with required skills to use all the equipment features foreseen for their respective role. - Trainees should be able to operate and maintain by themselves the equipment, as well as its components, at the end of the training period. - The Contractor will provide all the materials needed for training courses. Each trainee will receive individual hard copies of the technical books and documents used during courses. These will be transmitted to the Beneficiary at least two weeks before the beginning of respective sessions. The Contractor shall perform the following activities: 1. To develop and provide Training Plan with logistics and schedule for approval. 2. To conduct the at least one 5 days training with at least one (1) trainer for the six (6) end-users under this Contract 3. To prepare testing materials for the training stated above. 1.4 List of spare parts |  |  |  |  |  |

# Note:

# The tenderer financial offer contains all elements for Lot 1 presented in Annex II+III Technical specifications + Technical offer

DDP (Delivered Duty Paid)  — Incoterms 2010 International Chamber of Commerce http://www.iccwbo.org/products-and-services/trade-facilitation/incoterms-2010/the-incoterms-rules/ .